# Laboratory no. 5

# Arithmetical, logical, rotation and shift instructions for 80x86 microprocessor

The purpose of this worksheet is to present the arithmetical, logic, rotation and shift instructions for the microprocessor 80x86.

Theoretical approach

The 16 bits microprocessors from the 80x86 family have computing instructions to allow operations on 8 or 16 bits and to implement routines for multibytes or multiword operations. By computing we mean:

- arithmetical operations: add, subtract, multiply, divide, increment, decrement, complemental for 2,compare;

- logic operations: and, or, xor, complemental for 1;

- rotation and shift operations.

### Arithmetical instructions:

Arithmetical operations are using numbers in bytes or word format, with or without sign representation in complemental for 2. Add and subtract operations can also use operants of type BCD unpacked (one decimal digit by byte) or packed BCD (two decimal digits by byte). Multiply and divide operations can be used for unpacked BCD. In the following grid <s> and <d> represent the „source” operant and „destination” operant. Arithmetical instructions generally affect the following conditioning indicators: AF, CF, OF, DF, PF, ZF. These indicators are written according to the result of the instruction.

| General form | The effect | Affected indicators |
| --- | --- | --- |
| ADD <d>, <s> | <d>  {<d>}+{<s>} | AF,CF,OF,PF,SF,ZF |
| ADC <d>, <s> | <d>  {<d>}+{<s>}+{CF} | AF,CF,OF,PF,SF,ZF |
| INC <d> | <d>  {<d>}+1 | AF,OF,PF,SF,ZF |
| AAA | Decimal correction after adding in unpacked BCD (implicit AL) | AF,CF unmodified OF,PF,SF,ZF undefined |
| DAA | Decimal correction after adding in packed BCD (implicit AL) | AF,CF,PF,SF,ZF modified OF undefined |
| SUB <d>, <s> | <d>  {<d>}-{<s>} | AF,CF,OF,PF,SF,ZF |
| SBB <d>, <s> | <d>  {<d>}-{<s>}-{CF} | AF,CF,OF,PF,SF,ZF |
| CMP <d>, <s> | The indicators maintain the SUB positioning but the destination {<d>} doesn’t change | AF,CF,OF,PF,SF,ZF |
| DEC <d> | <d>  {<d>}–1 | AF,OF,PF,SF,ZF |
| NEG <d> | <d>  [0-{<d>}] | AF,CF,OF,PF,SF,ZF |
| AAS | Decimal correction after subtracting in unpacked BCD (implicit AL) | AF,CF modified OF,PF,SF,ZF undefined |
| DAS | Decimal correction after subtracting in packed BCD (implicit AL) | AF,CF,PF,ZF,SF modified OF undefined |
| CBW | Conversion from byte stored in AL to word stored in AX (sign extension) | --- |
| CWD | Conversion from byte stored in AX to double word stored in DX, AX (sign extension) | --- |
| MUL <s> | if <s> is a byte: AX  (AL)*{<s>} if <s> is a word: DX, AX  (AX)*{<s>} Operands are handled as unsigned integer | CF,OF modified AF,PF,SF,ZF undefined If CF and OF are 1 then AH (resp. DX) store values different from 0 |
| IMUL <s> | if <s> is a byte: AX  (AL)*{<s>} if <s> is a word: DX:AX  (AX)*{<s>} Operants are handled as sign integer. | CF,OF modified AF,PF,SF,ZF undefined If CF and OF are 1 then AH (resp. DX) store values different from 0 |
| AAM | Decimal correction after multiplying in BCD unpacked. MUL is used to multiply and then correction is set. AX stores the result. | SF,PF,ZF modified OF,AF,CF undefined |
| DIV <s> | if <s> is a byte: AL  [(AX)/{<s>}] AH  (AX) mod {<s>} if <s> is a word: AX  [(DX,AX)/{<s>}] DX  (DX,AX) mod {<s>} Operands are handled as unsigned integer. If the quotient exceeds destination’s capacity a level 0 interruption will be generated. | AF,CF,OF,PF,SF,ZF undefined |
| AAD | Decimal correction before a divide operation in BCD unpacked. The correction is made and then DIV is used for dividing. | PF,SF,ZF modified AF,CF,OF undefined |

For adding and incrementing are used the instructions ADD, ADC and INC according to syntax:

ADD {register | memory}, {register | memory | immediate data}

ADC {register | memory}, {register | memory | immediate data}

INC {register | memory}

For subtract and decrement operations are used SUB, SBB, DEC and NEG according to syntax:

SUB {register | memory}, {register | memory | immediate date}

SBB {register | memory}, {register | memory | immediate date}

DEC {register | memory}

NEG {register | memory}

Adding and subtracting can be interpreted as operations involving 2 numbers in complemental for 2 with or without sign representation. The developer of the program must choose how to represent the operands, how to evaluate the result properly and take efficient actions in case of overflowing.

For numbers without sign representation the overflow is signaled through written CF conditioning indicator.

Overflowing must be tested through conditioned jump instructions JC, JNC, JO, JNO or through the interruption instruction INTO, in order to call the routines for handling errors.

Example:

data 	segment

mem8 db 39

data 	ends

code 	segment

... ... 						with sign	unsigned

mov al, 26	;load al			26	26

inc al	;increment al		  	 1	1

add al, 76	;add immediate date    	76	76

;				---- 	----

;				103	103

add al, mem8;add memory 		  39	  39

;				---- 	----

;				142	-114+OF

mov ah, al	;copy to ah 			142

add al, ah	;add  register 		142

;				----

;				28+CF

... ...

code 	ends

For this example the add operation was on 8 bits. When the sum is over 127 the OF is written, when over 255 the CF is written.

Example:

data 	segment

mem8 db 122

data 	ends

code 	segment

… …

mov al, 95	;load  al				95		95

dec al	;decrement				-1		-1

sub al, 23	;subtract immediate value		-23		-23

----		----

71		71

sub al, mem8	;subtract memory		-122		-122

----		----

205+CF	-51

mov ah, 119;load ah

sub al,ah	;subtract register		 			-119

----

86+OF

… …

code 	ends

The instructions ADC and SBB allow implementation for multi-byte or multi-word operations. They perform the action of  ADD and SUB and also add or subtract the value of CF indicator.

Example:

data  	segment

mem32 dd 316423

data  	ends

code 	segment

… …

mov ax, 43981

sub dx, dx				;load dx, ax		            43981

add  ax, word ptr mem32[0]	;add inf. word

adc  dx, word ptr mem32[2]	;add sup. word 		316423

;				--------

;result in dx,ax	            	360404

… …

code 	ends

Example:

data 	segment

mem32a dd 316423

mem32b dd 156739

data 	ends

code 	segment

… …

mov ax, word ptr mem32a[0]	;load inf. word

mov dx, word ptr mem32a[2]	; load sup. word

sub ax, word ptr mem32b[0]		;subtract inf. word

sub ax, word ptr mem32b[2]		; subtract sup. word

… …

code 	ends

MUL is used for multiplying unsigned numbers. IMUL is used for multiplying numbers wtih sign representation. The syntaxes are :

MUL	{register | memory}

IMUL	{register | memory}

For multiplying instructions one of the operands must be loaded in the accumulator register (AL for 8 bits operands and AX for 16 bits operands). This is an implicit register, it is not specified in the instruction. The information stored in this register will be destroyed by the operation. The second operant must be specified as an operant in register or memory. This operant will not be destroyed by the operation unless it is DX, AH or AL. Multiplying two 8 bits numbers leads to a 16 bits result stored in AX. Multiplying two 16 bits numbers leads to a 32 bits result stored in DX, AX. For both cases if the superior half of the result is 0 (for unsigned mul.) or it coincides with the sign extension (for mul. in sign representation), the indicators CF and OF are set on 0; otherwise are set on 1. The other indicators have undefined values.

Example:

data 	segment

mem16 dw –30000

data 	ends

code 	segment

;unsigned multiply on 8 bits

mov al, 23		;load al			            23

mov bl, 24		;load bl			24

mul bl		;multiply bl

;				----

;result in ax			552

;positioning for CF and OF

;multiply with sign on 16 bits

mov ax, 50   	;load ax			50

imul mem16	;multiply with mem.		-30000

;				--------

;result in dx,ax		-1500000

; positioning for CF and OF

… …

code 	ends

DIV instruction is used for dividing unsigned numbers; IDIV is used for numbers without sign representation. The syntaxes are:

DIV	{register | memory}

IDIV	{register | memory}

In order to divide a 16 bits number by an 8 bits number the first operant is loaded in AX. This register looses it’s information after the operation ends. The first operant is specified as an 8 bits operant of register type or memory location. At the end AL stores the quotient and AH the rest.

In order to divide a 32 bits number by a 16 bits number the first operant is loaded in the pair DX, AX. The information stored in DX and AX will be lost after the operation. The first operant is specified as a 16 bits operant of register type or memory location. At the end AX stores the quotient and DX the rest.

For dividing 2 numbers of equal length (8 or 16 bits) the first action is to convert to a double length (16 or 32 bits) the first operant. For unsigned numbers the conversion consists in deleting the upper byte of the first operant, register AH, and respectively the most significant byte, register DX. For sign represented numbers conversion consists in sign extension and is obtained through CWB and CWD instructions.

If the divider is 0 or the quotient exceeds it’s assigned register (AL or AX) then the processor generates a level 0 interruption. If this interruption is not handled by the developer the operating system will abandon the program. There are two methods for dealing with the situation: testing the divider before the operation takes place and calling, when needed, a routine for handling errors; writing your own routine for handling errors to replace the routine for level 0 interruption.

Example:

data  	segment

mem16 dw –2000

mem32 dd 500000

data 	ends

code 	segment

; unsigned  dividing of a 16 bits operant

;by an 8 bits operant

mov ax, 700		;load  operant                 		700

mov bl, 36			;load divider                           		36

div bl			;unsigned dividing

;quotient is in al			19

;rest is in ah				16

;sign dividing of  a 32 bits operant

;by a 16 bits operant

mov ax, word ptr mem32[0]	;load ax

mov dx, word ptr mem32[2]	; load dx			500000

idiv mem16			; sign dividing

; quotient is in ax			-250

; rest is in dx				0

; sign dividing of  a 16 bits operant

;by a 16 bits operant

mov ax, word ptr mem16 	; load operant         			-2000

cwd					;convert to double word

mov bx, -421			; load divider    			-421

idiv bx				; sign dividing

; quotient is in ax			4

; rest is in dx				-316

code 	ends

### Operations in BCD unpacked:

The instruction set has 4 instructions for decimal correction: AAA (decimal correction after adding), AAS (decimal correction after subtracting), AAM (decimal correction after multiplying) and AAD (decimal correction before dividing), which allow arithmetical operations for BCD unpacked. For this purpose classic arithmetical instructions are used, followed (preceded) by an instruction to correct the result. Arithmetical operations must apply on bytes so the result can be found in AL, which is used by correction instructions. If an operation implies 2 one digit operands with a result of two digits, the instruction for correction will place the least significant digit in AL, and the most significant in AH, deleting the content of this register. If the digit stored in AL generates carry to AH or needs to borrow from AH the indicators CF and AF are positioned.

Example:

;Adding in BCD unpacked

mov ax, 9		;load ax				0009h

mov bx,3		; load bx				0003h

add al, bl		;adding				000ch

aaa			;decimal correction for add			ax=0102h

; AF and CF are positioned

;Subtract in BCD unpacked

mov ax, 0103h		;load ax				0103h

mov bx, 4		; load bx				0004h

sub al, bl		;subtract				01feh

aas			; decimal correction for

;subtraction				ax=0009h

;AF and CF are positioned

;Multiply in BCD unpacked

mov ax, 0903h		; load ax				0903h

mul ah                	;unsigned multiply 			001bh

aam                     	; decimal correction after MUL 	ax=0207h

;Dividing in BCD unpacked

mov ax, 0209h     	; load în ax			0209h

mov bl, 2              	; load bl				02h

aad                      	; decimal correction before

;dividing 				AX=0019h

div bl                   	;unsigned dividing 			010ch

;quotient in al				0ch

;rest in ah 				01h

aam                       	;decimal correction after

;dividing the quotient			ax=0102h

;the rest is lost

The rest will be lost. If needed, it must be saved in a different register before correcting the quotient. The rest can also be corrected.For this it must be transfered in AL.

### Operations in BCD packed:

The instruction set has two instructions for decimal correction DAA (decimal correction after add) and DAS (decimal correction after subtract) wich allow adding and subtracting in packed BCD. For this are used classic instructions to add and subtract followed by appropriate instructions to correct the result.

Arithmetical operations must be on bytes in order to store the result in AL..

Instructions for decimal corrections in packed BCD never affect AH register. AF indicator is positioned in case of carry or borrow from the least significant digit to the most significant one. CF indicator is positioned in case of carry or borrow to exterior.

Example:

;Adding in packed BCD

mov ax, 8833h		;load ax				 8833h

add al, ah                	;add to al    			            al=0bbh

daa                         	;decimal correction

;after adding				al=021h

; CF is set

;the result is	 121h

;Subtracting in packed BCD

mov ax, 3883h       	;load ax				3883h

sub al, ah		;subtract    	 				al=04bh

das                          	;decimal correction

;after subtracting				al=045h

; CF is 0

### Logic instructions:

| General form | Effect | Affected conditioning indicators |
| --- | --- | --- |
| AND <d>, <s> | <d>  {<d>} and {<s>} | CF,OF,PF,SF,ZF modif. AF undefined |
| TEST <d>, <s> | The indicators are set as for AND but {<d>} does not change | CF,OF,PF,SF,ZF modif. AF unmodified |
| OR <d>, <s> | <d>  {<d>} or {<s>} | CF,OF,PF,SF,ZF modif. AF undefined |
| XOR <d>, <s> | <d>  {<d>} xor {<s>} | CF,OF,PF,SF,ZF modif. AF undefined |
| NOT <s> | <s>  not <s> (complemental for 1) | ---- |

Logic instructions operate on bits, over bits of same rank of two operants.There are 5 logic instructions: AND, TEST, OR, XOR and NOT.

The syntax:

AND	{register | memory}, { register | memory | immediate date}

TEST	{ register | memory }, { register | memory | immediate date }

OR	{ register | memory }, { register | memory | immediate date }

XOR	{ register | memory }, { register | memory | immediate date }

NOT	{ register | memory }

Example:

;example for  AND

mov al, 35h 	;load al					00110101

and al, 0fbh       	;and with immediate date		11111011

;					------------						;					00110001

and al, 0f8h	;					11111000

;					------------

;					00110000

;example for OR

mov al, 35h       	;load al			  		00110101

or al, 08h           	;or with immediate date		00001000

;					------------

or al, 07h		; or with immediate date		00000111

;					------------

;					00111111

;example for XOR

mov al, 35h      	;load al					00110101

xor al, 08h        	;xor with immediate date	 	00001000

;					------------

;					00111101

xor al, 07h            	; xor with immediate date     		00000111

;					------------

;					00111010

Logic instructions can be used to compare an operant with 0(or bx, bx instead of cmp bx, 00) or to delete an operant (xor cx, cx; sub cx, cx în loc de mov cx, 00) having a more compact form.

### Shift and rotation instructions:

| General form | Effect | Affected conditioning indicators |
| --- | --- | --- |
| SHL <s>, 1 SAL <s>, 1 | Logic shift to left CF will store the most significant bit that was shifted. If <CF> <> the initial sign OF becomes 1. | CF,OF,SF,ZF,PF AF undefined |
| SHL <s>, CL SAL <s>, CL | Logic shift to left with a number of positions indicated by CL. CF will store the last shifted bit. | CF,OF,SF,ZF,PF AF undefined |
| SHR <s>, 1 | Logic shift to right. Zeroes are inserted. CF will store the most significant bit. If the most significant bits of the result are different OF becomes 1. | CF,OF,SF,ZF,PF AF undefined |
| SHR <s>, CL | Logic shift to right with a number of positiones indicated by CL. CF will store the last shifted bit. | CF,OF,SF,ZF,PF AF undefined |
| SAR <s>, 1 | Arithmetic shift to right. Sign extension. The least significant bit will be stored by CF. If the most significant bits of the result are different OF becomes 1. | CF,OF,SF,ZF,PF AF undefined |
| SAR <s>, CL | Arithmetic shift to right with a number of positiones indicated by CL. CF will store the last shifted bit. | CF,OF,SF,ZF,PF AF undefined |
| ROL <s>, 1 | Rotate left by carry. If CF <> sign then OF becomes 1 | CF, OF |
| ROL <s>, CL | Rotate left by carry with a number of positions indicated by CL. | CF, OF |
| ROR <s>,1 | Rotate right by carry. If (CF) <> sign OF becomes 1. | CF, OF |
| ROR <s>, CL | Rotate right by carry with a number of positions indicated by CL. | CF, OF |
| RCL <s>, 1 | Rotate left with carry. If (CF) <> sign OF becomes 1. | CF, OF |
| RCL <s>, CL | Rotate left with carry with a number of positions indicated by CL. | CF, OF |
| RCR <s>, 1 | Rotate right with carry. If (CF) <> sign OF becomes 1. | CF, OF |
| RCR <s>, CL | Rotate right with carry with a number of positions indicated by CL. | CF, OF |

The format for all shift and rotate instructions is identical:

MNEMONICA_INSTR_DEPL_ROT {register | memory}, {CL | 1}

Destination operant must contain the address that will be shifted (register or memory location).After the instruction is executed here will be stored the shifted/rotated value. Source operant must contain the number of bits over witch the shift/rotation takes place. It can be the immediate value 1 or register CL

The following figures show the result of these instructions on a byte operant for one position shifting/rotation.

SHL, SAL

Carry	        7          6          5         4          3          2          1          0

0

SHR

Carry	        7          6          5         4          3          2          1          0

0

SAR

Carry		7          6          5         4          3          2          1          0

s	ROL

Carry	        7          6          5         4          3          2          1          0

ROR

Carry	        7          6          5         4          3          2          1          0

RCL

Carry	        7          6          5         4          3          2          1          0

RCR

Carry	        7          6          5         4          3          2          1          0

Example:

;a number stored in ax

;is multiplied by 10

shl ax, 1		;*2

mov bx, ax    	;

shl ax, 1		;*4

shl ax, 1	 	;*8

add ax,	 bx	;*10

;an unsigned number stored in ax

;is divided by 512

shr ax, 1		;/2

xchg ah, al	;

cbw 			;/512

;a number stored in ax represented in C2 with sign

;is divided by 2

mov ax, -16		;

sar ax, 1           	;/2

;a 32 bits unsigned number

;is divided by 2

data 	segment

mem32 dd 500000

data 	ends

code 	segment

… …

shr word ptr mem32[2], 1	;shifting in CF

rcr word ptr mem32[0], 1	;rotation with CF

… …

code 	ends

## Laboratory tasks

- Study the examples.

- Follow the examples with Turbo Debugger.

- Write a program that generates an integer in byte representation and stores it to a REZ location after the formula:

REZ = al*num1+(num2*al+bl)

All parameters are byte represented.

- Implement the following operations using arithmetic and shift instructions:

AX = 7*ax–2*bx–bx/8

Parameters are byte represented.

- Design an algorithm to multiply and divide two unsigned/signed numbers represented in C2 on 1 and on 2 bytes.

- (complementary) Design an algorithm to multiply two 4 bytes numbers in C2 representation.