# Laboratory Work no. 7

# Flow control instructions

## Object of laboratory

The 8086 family microprocessors have a large variety of instructions that allows the instruction flow control. They divide in 4 categories: jump, cycling, calling and break instructions.

## Theoretical considerations

### Jump instructions.

Jumping is the most direct method of modifying the instruction flow. At the internal level the jump instructions work by changing the value of the IP register and sometimes of the CS register (between segments jumping), so the next instruction address that exists in this registers will be changed with the destination address.

### The unconditional jump.

The JMP instruction is used for making an unconditional jump to a specified address. The jump between segments can be short or near, regarding the destination address that can be between -127..127 of the current address or is situated just in the same segment. The jump between segments is far type.

From the destination address specification point of view there are direct and indirect jumps. In the direct jumps, the destination address is specified through a label. The syntax is:

JMP	label

For the jumps in the same segment, the addressing is IP relative.

After the instruction code there is a displacement on an octet or two octets that represent the distance from the current address to the destination.

Example:

ALFA:

…

JMP	ALFA

If the label is situated under 128 octets far from the jump instruction and has been defined before, than a short jump type is codified. If the label is defined after the jump instruction than a near jump type is codified, indifferent if the distance between the jump instruction and label is lesser or not than 128 octets. In this situation for obtaining a short codification type there can be used the SHORT operator.

Example:

JMP	SHORT BETA

…

BETA:

Observation: Using the SHORT operator in an improper situation is signalized as an error.

In case of using the direct addressing for doing jumps between segments after the instruction code is followed by a displacement of four octets that represent the destination address.

If the label has been defined before, the codifying is correct. If the label is being defined afterwards, is necessary to specify the FAR type for this label when calling.

Example:

JMP	FAR PTR GAMA

…

GAMA:

In case of indirect jumping, the destination address is specified through an operand, the syntax is:

JMP {register| memory}

Example:

JMP	AX

JMP	[BX]

JMP	ALFA		; ALFA is a var. word or double word

If the variable is defined after the jump instruction in the case of between segment jumping there must be used DWORD PTR operator.

Example:

JMP	DWORD PTR ALFA

An unconditional jump can be used as ca conditional jump form if the destination

address is specified in a register or a memory location.

The destination address can be calculated while executing the program based on the interaction with the user or other factors.

Example:

code	segment

jmp		proces

ctl_tbl	LABEL	WORD

dw		extended		; the key with extended code(2 car.)

dw		ctrla		; the key CTRL/A

dw		ctrlb		; the key CTRL/B

proces:

mov		ah, 8h		; reading the key in AL

int		21h

cbw

mov		bx, ax

shl		bx, 1			; the address calculation in the table

jmp		ctl_tbl[bx]

…

extended:

mov		ah, 8h		; takes the second cod

int		21h

…

ctrla:					; routine for CTRL/A

…

jmp		next

ctrlb:					; routine for CTRL/B

…

jmp		next

next:						; continue

…

code	ends

### Conditional jumps

The conditional jump is the most frequent method of modifying the instruction flow. It consists of a process in two steps. In the first step the condition is tested and in the second the jump is done if the condition is true or the next instruction in executed if the condition is false

The jump instruction syntax is:

Jcondition	label

The conditional jumps are short type, so that the distance at the destination address must be lesser than 128 octets. In a contrary case the error is signaled. The destination address is specified through a displacement on an octet of the current address. The conditional jumps use as a condition the indicators state or combinations of those.

The testing step is realized with the help of the instructions that affect the condition indicators. In this purpose the most frequent are used the CMP and TEST instructions.

The jumping step is made using one of the 13 conditional jump instructions.

If one conditional salt must be greater than 128 octets, it must be replaced through a conditional jump of reverse condition followed by an unconditional jump.

Example:

cmp	ax, 7

je	NEAR

cmp	ax, 6		; if AX is 6 and the jump is greater than 128 octets

; the instruction of conditional jumping is replaced

jne	NEAR

jmp	FAR

…

NEAR:			; less than 128 octets

; from the jump instruction

…

FAR:				; more than  128 octets

; from the jump instruction

### Compare and jump

The CMP instruction compares 2 operands through substraction of the operand source from the destination operand without affecting the destination and with the proper entry of the condition indicators. The syntax:

CMP   {register memory, {register | memory | immediate value}

It is used for testing the next relations: equal, not equal, greater than, less than, greater or equal than, less or equal than.

The conditional jump instruction used after the compare instruction has the flow chart accordance with the tested relation, generated form the next letters:

LETTER					SEMNIFICATION

J					Jump

G					Greater than	(for no. with sign)

L					Less than	(for no. with sign)

A					Above 		(for no. without sign)

B					Below 		(for no. without sign)

E					Equal

N					Not

In the next table there are represented the conditional jump instruction according to each relation:

| Jump condition | Compare with sign | Jump condition | Compare without sign | Jump condition |
| --- | --- | --- | --- | --- |
| Equal = | JE | ZF=1 | JE | ZF=1 |
| Not equal <> | JNE | ZF=0 | JNE | ZF=0 |
| Greater than > | JG or JNLE | ZF=0 and SF=OF | JA or JNBE | ZF=0 and CF=0 |
| Less than < | JL or JNGE | SF<>OF | JB or JNAE | CF=1 |
| Greater than or equal >= | JGE or JNL | SF=OF | JAE or JNB | CF=0 |
| Less than or equal <= | JLE or JNG | ZF=1 or SF=OF | JBE or JNA | CF=1 or ZF=1 |

Example

; IF (CX<-20) THEN DX=30 ELSE DX=20

cmp	cx, -20

jl 	less

mov	dx, 20

jmp	cont

less:

mov	dx, 30

cont:

Example:

; IF (CX>=-20) THEN DX=30 ELSE DX=20

cmp	cx, -20

jl 	notless

mov	dx, 20

jmp	cont

less:

mov	dx, 30

cont:

Jumps based on the conditional pointers state:

| INSTRUCTIONS | JUMP CONDITIONS |
| --- | --- |
| JO JNO JC JNC JZ JNZ JS JNS JP JNP JPE JPO JCXZ | OF=1 OF=0 CF=1 CF=0 ZF=0 ZF=1 SF=1 SF=0 PF=1 PF=0 PF=1 PF=0 CX=0 |

As it can be observed JCXZ is the only conditional jump instruction that does not test the conditional pointers but the content of the CX register.

Example:

add	ax, bx

jo	overflow

…

overflow:

### Cycling instructions

The cycling instructions allow an easy programming of the control structures of the final test cycle type.

The syntax of these instructions is:

LOOP	label 		; CX is decremented and if CX is not null the jump is                            ; done.

LOOPE	label   		; CX is decremented and if CX is not null and ZF=1 the                           ; jump is done.

LOOPZ	label   		; identical with LOOPE

LOOPNE	label   		; CX is decremented and if CX is not null and ZF=1 the                           ; jump is done.

LOOPNZ 	label		; identical with LOOPNE

The cycling instructions decrement the content of the CX register and if the jump condition is fulfilled the jump is done.

The distance between the looping instruction and the destination address must be between the interval -128..127 octets

Example:

mov	cx, 200		; initialize counter

next:

…

loop	next			; repeat if CX in not null

; continue after the cycle

This loop has the same effect as the one in the next example:

Example:

mov	cx, 200

next:

…

dec	cx

cmp 	cx, 0

jne 	next

The first version is more efficient.

Using the JCXZ instruction allows us to realize some control instructions of the starting test cycle type.

Example:

next:

jcxz	cont

…

loop	next

cont:

### Using the procedures

### The procedures are code units that execute specific functions. They represent a way of modulating the code so that a specific function can be executed from any other point in the program without having to introduce each block.

The procedures from the assembler language are comparable with the C functions, the subprograms, the functions and the subroutine from BASIC, subroutine and the functions from FORTRAN etc.

For defining and using some procedures there are two directions and instructions. So the PROC and ENDP directions mark the beginning and the end of defining the procedure. The CALL instruction is used for calling the defined procedures, and the RET instruction is used for controlling the transfer to the caller.

The CALL and RET instructions use the stack for saving and retrieval the address of return. The CALL instruction introduces in stack the return address (the address of the next CALL instruction) and then the jump to the address at the beginning of the procedure is done.

The RET instruction extracts from the stack the address introduced by the CALL instruction and returns the instruction control of the next immediate called address.

The procedures can be found or not in the same segment with the called instructions.

From this point of view there are NEAR and FAR type procedures. When declaring the procedures their type is declared too. The NEAR type is implicit.

The procedure definition syntax is:

Label		PROC	[NEAR | FAR]

…

RET	[constant]

Label		ENDP

The RET instruction allows one constant operand that specify a number of octets that will be added to the content of the SP register after returning from the procedure. This operand can be used for deleting from the stack the arguments that were transmitted to the procedure through the stack.

The calling procedure syntax is:

CALL   {register | memory}

## Lab tasks

- There will be studied the instructions and the examples presented before.

- There will be written a program sequence that transforms the code of a small letter in the code of the big letter. The code will be taken from one memory location and it will be putted in the same memory location.

- Write a program that calculates the arithmetic average of the numbers from a set that has values between 5 and 10. Write the average obtained on the display plotting “The average is: 8”. The average will be calculated as a whole number, and for the interruptions of printing data on the display The Norton Guide functions: will be studied.

- Write a program that plots an integer without sign from the AX register. Indication: more divides by 10 will be made, and the obtained digits will be printed in the reverse order.

- Write program that reads an integer without sign from the keyboard until the enter key is pressed. Indication: every digit you read will be converted to its numeric value and the writing  model of writing a number will be used like this: 145=(((1*10)+4)*10)+5

- Write a procedure that converts a hex digit (00-0F) in an ASCII character. Transmitting the hex digit to the procedure is done in the AL register, and the procedure returns the ASCII character in the same register. Write the procedure calling for more cases and test.

- Write a procedure for converting an ASCII character into a hex digit. The conversion of the ASCII character (to the procedure) and the hex digit (to the calling program) in done through the AL register. The procedure will set the CF indicator in the case where the ASCII character does not correspond to a hex digit.

- Write a procedure for comparing two sets of ASCII characters, found in different segments. The address of the sets is delivered to the procedure in registers. At exit, AL will contain 00 if the sets are identical and FF if the sets are different.